# GETINCELL-LINE® | Double blade louvre – mounted on T15/T24 carrier

Double blade open cell ceiling consisting of a substructure [T-shaped rail] and prefabricated panel elements.

The T-shaped rail is suspended rigidly from the mounting level [e.g. bare ceiling] via a suspension hanger for threaded rod with M6 threaded rods using technical approved dowels [distance of the suspensions according to EN 13964]. The distance between the T-profiles depends on the specified module dimension – according to the panel size length.

The panel elements are mounted at the module spacing. The individual panels consist of longitudinal bars and crossbars in square cells and are self-adjusting. The individual panel elements are placed on the T-profiles [visible construction].

Typically, the edge connection is carried out by means of a shadow gap.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – GETINCELL-LINE® double blade open cell ceiling Panel size [LxW]:  600x600mm  625x625mm  1200x600mm  1250x625mm Blade profile width/height:  4/42mm  9/40mm Cell size: ............... mm [51-187mm] Perforation:  Microperforation [Ø=1.5mm; diagonal pitch]  Perfo 2.5 [Ø=2.5mm; diagonal pitch] Exterior paint:  White  Black  RAL 9006  RAL 7012 Other colours on request Interior paint:  Green  Blue [on request]  Fuchsia [on request] Other colours on request Material: Aluminium [edges not coated] Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. m² | T-shaped rail [T15/T24] Colour:  White Other colours on request Unit price: …............ €/m² Total price: ............… € |