# GEOTE® | Double blade louvre – mounted on T15 carrier

Double blade open cell ceiling consisting of a substructure [T-shaped rail] and prefabricated panel elements.

The T-shaped rail is suspended rigidly from the mounting level [e.g. bare ceiling] via a suspension hanger for threaded rod with M6 threaded rods using technical approved dowels [distance of the suspensions according to EN 13964]. The distance between the T-profiles depends on the specified module dimension – according to the panel size length.

The panel elements are mounted at the module spacing. The individual panels consist of longitudinal bars and crossbars in square cells and are self-adjusting. The individual panel elements are placed on the T-profiles [visible construction].

Typically, the edge connection is carried out by means of a shadow gap.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – GEOTE® double blade open cell ceiling Panel size [LxW]:  600x600mm  625x625mm  1200x600mm  1250x625mm Blade height/width/:  37/15mm  47/15mm  57/15mm Cell size:  ............... mm [600mm panel: 50-200mm; precise dimensions on request]  ............... mm [625mm panel: 52-208mm; precise dimensions on request] Other dimensions on request Colour:  White  Black  RAL 9006 Other colours on request Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. m² | T-shaped rail [T15] Colour:  White Other colours on request Unit price: …............ €/m² Total price: ............… € |